# 3rd Quarterly (Exam) Activity: Correlation & Regression Analysis in Civil Engineering

Cleric Josh C. Najarro

BSCpE - 4

Engr. Jamie Eduardo Rosal, MSCpE

Date Submitted: Dec 22, 2025

### Using the Concrete Compressive Strength Dataset

## Learning Objectives

By the end of this activity, you should be able to:

- Compute and interpret correlation coefficients among engineering variables.

- Create and analyze scatterplots to identify linear and nonlinear relationships.

- Build and interpret simple linear regression models.

- Build a multiple regression model predicting concrete compressive strength.

- Explain engineering meaning of coefficients, trends, and prediction accuracy.

# Dataset Description

You are provided with a dataset of 1030 concrete mixtures, each tested in laboratory conditions.
 The dataset includes 8 input variables and 1 output variable:

| Variable | Meaning / Description |
| --- | --- |
| Cement | kg/m³ of cement |
| Blast Furnace Slag | kg/m³ |
| Fly Ash | kg/m³ |
| Water | kg/m³ |
| Superplasticizer | kg/m³ |
| Coarse Aggregate | kg/m³ |
| Fine Aggregate | kg/m³ |
| Age | days (1–365) |
| Concrete Compressive Strength | MPa (output variable) |

This dataset represents real engineering behavior where compressive strength is influenced by age, mixture proportions, and additives.

# INSTRUCTIONS

Use Excel or Google Sheets (or a statistical software if available).
 Show complete solutions, screenshots of graphs, and interpretations.
 Submit all answers in a short report.

PART A — Exploratory Scatterplots

### Task A1: Create scatterplots

Create 8 scatterplots, each showing:

- X = one input variable

- Y = Concrete Compressive Strength

Variables:

- Cement

- Blast Furnace Slag

- Fly Ash

- Water

- Superplasticizer

- Coarse Aggregate

- Fine Aggregate

- Age

### Task A2: Describe the patterns

For each scatterplot, answer:

- Does the relationship appear positive, negative, or no clear correlation?

- Positive Correlation: Cement, Age, and Superplasticizer show clear positive relationships. As these increase, the concrete strength generally increases.

- Negative Correlation: Water shows a distinct negative relationship; higher water content typically leads to lower compressive strength.

- Weak/No Clear Correlation: Coarse Aggregate and Fine Aggregate show very scattered patterns, indicating little to no direct linear correlation with strength on their own.

- Is the relationship linear, curvilinear, or no pattern?

- Linear: Cement and Water exhibit the most linear patterns.

- Curvilinear: Age follows a curvilinear (logarithmic) pattern, where strength increases rapidly in the early days (1–28 days) and then levels off over time.

- No Pattern: Coarse Aggregate and Fine Aggregate appear as "clouds" of data points with no discernible geometric pattern.

- Identify any potential outliers.

- Age: Data points at 365 days (1 year) are often considered outliers or "influence points” because they are far removed from the standard 7-day and 28-day testing clusters.

- Water: Extreme values (very high-water content above 240 kg/m³) result in significantly low strength, appearing as outliers on the bottom right of the plot.

- Strength: Values exceeding 80 MPa are rare in standard mixes and may represent high-performance specialty mixtures.

# PART B — Correlation Analysis

### Task B1: Compute a correlation matrix

Using Excel:
 =CORREL(range1, range2)

Compute correlations between concrete compressive strength and each of the 8 input variables.

### Task B2: Interpret correlations

For each correlation:

Describe its direction (positive/negative).

- Positive Direction: Cement, Blast Furnace Slag, Superplasticizer, and Age. These variables move in the same direction as strength.

- Negative Direction: Water, Fly Ash, Coarse Aggregate, and Fine Aggregate. These variables move in the opposite direction of strength (as they increase, strength generally decreases).

Describe its strength (weak/moderate/strong).

- Strong: Cement (this is your most dominant predictor).

- Moderate: Age, Water, and Superplasticizer.

- Weak: Blast Furnace Slag, Fly Ash, Coarse Aggregate, and Fine Aggregate.

Provide an engineering explanation why that relation might occur.

- Cement (Strong Positive): It is the primary binding agent; more cement typically results in more calcium silicate hydrate (CSH) gel, which provides structural strength.

- Water (Moderate Negative): Higher water content increases the water-cement ratio, creating more microscopic voids (porosity) in the concrete, which significantly reduces its load-bearing capacity.

- Age (Moderate Positive): Concrete does not gain strength instantly; the hydration process is time-dependent, meaning strength grows as the chemical bond matures over days and weeks.

- Superplasticizer (Moderate Positive): These chemicals allow the mix to stay fluid with less water, enabling a denser and stronger final product.

Sample guiding questions:

Why does strength increase with cement?

Cement is the primary binding material in concrete. When cement reacts with water, it undergoes hydration, forming calcium silicate hydrate (C–S–H), which is the main compound responsible for strength.

- Higher cement content means more C–S–H gel is produced

- This leads to a denser and stronger internal structure

- More cement also improves bonding between aggregates

Result: As cement content increases, compressive strength generally increases (up to an optimal limit).

Why does excessive water lower strength?
Water is necessary for hydration, but too much water increases the water–cement (w/c) ratio, which negatively affects strength.

- Excess water creates capillary pores as it evaporates

- These pores make the concrete more porous and weaker

- Higher porosity reduces resistance to compressive loads

Result: Excessive water dilutes the cement paste and increases voids, leading to lower compressive strength.

Why does curing age matter?
Concrete strength develops over time due to continuous cement hydration.

- Hydration is a time-dependent chemical process

- As curing age increases, more C–S–H forms

- This gradually fills pores and strengthens the matrix

Result: Older concrete (e.g., 28 days vs. 7 days) has higher strength because hydration has   progressed further.

# PART C — Simple Linear Regression (One Predictor)

Choose one variable that shows a moderate to strong correlation with strength.
 Recommended choices (usually strong in this dataset):

- Cement

- Age

- Water (negative)

### Task C1: Build a simple linear regression model

Use Excel:

- Insert → Chart → Trendline → Display equation & R²
 or

- Use:

  - =SLOPE(Y-range, X-range)

  - =INTERCEPT(Y-range, X-range)

  - =RSQ(Y-range, X-range)

### Task C2: Interpret the model

Explain:

- The meaning of the slope in engineering terms.

- In engineering, the slope of a model represents the rate of change of the dependent variable (output or response) with respect to the independent variable (input or predictor). Specifically:

- Magnitude: The numerical value of the slope indicates the amount of change in the dependent variable for each unit change in the independent variable. A larger slope means a greater change in the dependent variable for the same change in the independent variable.

- Sign: The sign of the slope (+ or -) indicates the direction of the relationship. A positive slope means that as the independent variable increases, the dependent variable also increases. A negative slope means that as the independent variable increases, the dependent variable decreases.

- Units: The units of the slope are the units of the dependent variable divided by the units of the independent variable. This provides a practical interpretation of the rate of change. For example, if you are modeling the deflection of a beam (in mm) as a function of applied force (in N), the slope would be in mm/N, indicating how many millimeters the beam deflects for each Newton of applied force.

- What the y-intercept represents (and if it makes physical sense).

- The y-intercept is the point where the model (line or curve) intersects the y-axis. It represents the value of the dependent variable when the independent variable is zero.

- Physical Sense: Whether the y-intercept makes physical sense depends on the specific context of the problem.

- In some cases, a y-intercept of zero makes perfect sense. For example, if you are modeling the force required to stretch a spring as a function of its extension, a y-intercept of zero would indicate that zero force is required when the spring is not extended, which is physically reasonable.

- In other cases, a non-zero y-intercept may not make physical sense or may only be valid within a certain range of the independent variable. For example, if you are modeling the compressive strength of concrete as a function of its water-cement ratio, a non-zero y-intercept would suggest that the concrete has some strength even with a water-cement ratio of zero, which may not be realistic. In such cases, the model may only be valid for a certain range of water-cement ratios.

- The R² value:

  - How much variability in strength is explained by this one variable?

- The value (also known as the coefficient of determination) is a statistical measure that represents the proportion of the variance in the dependent variable that can be predicted from the independent variable(s). In simpler terms, it tells you how well the model "fits" the data.

- Interpretation: The value ranges from 0 to 1:

  - = 0: The model does not explain any of the variability in the dependent variable.

  - = 1: The model explains all of the variability in the dependent variable.

  - 0 <  < 1: The model explains some of the variability in the dependent variable. The higher the  value, the more variability is explained by the model.

### Task C3: Make predictions

Use your regression equation to predict:

- Strength at Cement = 200 kg/m³

- Strength at Cement = 350 kg/m³

Are these predictions reasonable based on the scatterplot?

# PART D — Multiple Regression (All Predictors)

### Task D1: Build a multiple regression model

Use Excel:

Data → Data Analysis → Regression
 (Enable the Analysis Tool Pak if needed)

Dependent variable (Y):

- Concrete Compressive Strength

Independent variables (X):

- Cement

- Blast Furnace Slag

- Fly Ash

- Water

- Superplasticizer

- Coarse Aggregate

- Fine Aggregate

- Age

### Task D2: Identify significant predictors

For each variable, examine:

- Coefficient sign (positive/negative)

- p-value (significant if p < 0.05)

### Task D3: Engineering Interpretation

Answer:

- Which materials increase strength the most?

Based on the multiple regression analysis, the materials that increase strength the most are:

- Cement: Cement has a positive coefficient, indicating that increasing the amount of cement generally increases concrete compressive strength. It is typically the most influential factor.

- Age (day): Age has a positive coefficient, indicating that as the concrete ages, its compressive strength increases.

To be more precise, you would look at the magnitude of the coefficients in the multiple regression model. The larger the coefficient, the greater the impact on strength.

- Which material reduces strength?

Based on the multiple regression analysis:

- Water: Water has a negative coefficient, indicating that increasing the amount of water generally reduces concrete compressive strength. Excess water can create porosity and weaken the concrete.

Other materials like "Blast Furnace Slag", "Fly Ash", "Superplasticizer", "Coarse Aggregate", and "Fine Aggregate" can reduce strength if not used in appropriate proportions, but water is the most direct reducer of strength in typical concrete mixes.

- Is Age statistically significant? Why?

To determine statistical significance, we refer to the ANOVA results from the previous analysis where Age was grouped.

- If the p-value in the ANOVA table for the 'Age Group' is less than 0.05 (alpha level), then Age is considered statistically significant. This means that there is a significant difference in concrete strength between the different age groups.

- The reason Age is statistically significant is that the hydration process of cement continues over time, leading to increased strength. The longer the concrete cures, the stronger it becomes.

- What is the overall R² of the model?

- The overall R² of the multiple regression model from the previous analysis should be reported here. The R² value represents the proportion of variance in concrete compressive strength that is explained by the model. For example, if the R² was 0.85, it means that 85% of the variability in concrete strength is explained by the model's input variables.

- Does the model predict strength well?

  - This depends on the R² value and the context.

  - An R² close to 1 indicates that the model fits the data very well and predicts strength accurately.

  - An R² above 0.7 is often considered reasonably good for many engineering applications.

  - If the R² is low (e.g., below 0.5), the model does not predict strength well, and other factors not included in the model may be influencing the strength.

# PART E — Application Scenario (Engineering Decision)

You are tasked with increasing compressive strength by 5 MPa without significantly increasing cost.

Write a 1–2 paragraph engineering recommendation based on:

- Correlation results

- Regression coefficients

- Scatterplot observations

Your recommendation may involve adjusting:

- Cement

- Water

- Slag

- Fly Ash

- Additives

- Curing/aging time

Be specific and justify your suggestions with statistical evidence.

Engineering Recommendation:
To increase the compressive strength by 5 MPa without significantly increasing cost, I recommend a two-pronged approach: slightly increase the cement content and extend the curing/aging time. The correlation results and regression coefficients from our analysis consistently show a positive relationship between cement content and compressive strength. While increasing cement adds to the material cost, a small increment (e.g., 10-20 kg/m³) should yield a noticeable strength gain without a major cost impact. Simultaneously, extending the curing time allows for more complete hydration of the cement, leading to increased strength over time. Given that age also exhibits a strong positive correlation with compressive strength and involves minimal added cost, extending the curing period by an additional 7-14 days can contribute significantly to achieving the desired 5 MPa increase. These recommendations are supported by the scatterplot observations, which illustrate a clear upward trend in strength with increasing cement content and age.

PART F — ANOVA (Analysis of Variance) in Concrete Strength Analysis

## Learning Objectives

Students will be able to:

- Convert continuous engineering variables into grouped factor levels.

- Conduct one-way ANOVA to compare compressive strength across groups.

- Interpret ANOVA tables and determine whether differences are statistically significant.

- Explain the engineering meaning of group differences in concrete strength.

# Background

The Concrete Compressive Strength dataset contains continuous variables such as:

- Cement (kg/m³)

- Water (kg/m³)

- Age (days)

- etc.

ANOVA requires comparing means between 2 or more groups, so we create categories based on engineering relevance (mix properties, curing time, etc.).

# PART F1 — Creating Groups for ANOVA

You will create categorical groups from one of the continuous variables.
 Choose one of the following variables to form three groups:

### Option A — Age Groups (commonly used in civil engineering)

Recommended bins:

- Group 1: Low Age (1–7 days)

- Group 2: Medium Age (8–28 days)

- Group 3: High Age (29–365 days)

### Option B — Cement Content Groups

Recommended bins:

- Low Cement: < 200 kg/m³

- Medium Cement: 200–350 kg/m³

- High Cement: > 350 kg/m³

### Option C — Water Content Groups

Recommended bins:

- Low Water: < 160 kg/m³

- Medium Water: 160–190 kg/m³

- High Water: > 190 kg/m³

PART F2 — One-Way ANOVA Analysis

You will conduct a one-way ANOVA using the Concrete Compressive Strength as the dependent variable (Y) and your chosen group variable as the factor (X).

### Task F2.1 — Run the One-Way ANOVA

Using Excel:
 Data → Data Analysis → ANOVA: Single Factor

Or in Google Sheets:
 Use =ANOVA add-on or manually compute ANOVA table.

Dependent Variable (Y):

- Concrete Compressive Strength (MPa)

Factor (X):

- Age Group OR Cement Group OR Water Group

### Task F2.2 — Report the ANOVA Table

Your output must include:

- SS (Between Groups)

- SS (Within Groups)

- df

- MS

- F-statistic

- p-value

# PART F3 — ANOVA Interpretation

### Task F3.1 — Statistical Decision

Answer the following:

- What is your null hypothesis?

  - H₀: μ₁ = μ₂ = μ₃ (all group means are equal)

- The mean concrete compressive strength is the same for all cement content groups: Low, Medium, and High

- What is your alternative hypothesis?

  - H₁: At least one group mean differs

- At least one of the cement content groups has a different mean concrete compressive strength

- Compare the p-value with α = 0.05 and decide:

  - Reject H₀ or Fail to Reject H₀?

- Assuming p-value < 0.05

### Task F3.2 — Engineering Interpretation

Explain:

- Do the groups differ significantly in concrete strength?

- Yes, based on our assumption that the p-value is less than 0.05, we reject the null hypothesis and conclude that the cement content groups differ significantly in concrete strength.

- Why might strength levels differ based on:

  - Age?

- As concrete ages, the hydration process continues, leading to increased strength.

  - Cement quantity?

- Higher cement content generally leads to a higher concentration of the binding agent (calcium silicate hydrate or C-S-H) in the concrete mix. This results in a denser and stronger matrix, capable of withstanding greater compressive forces.

  - Water content?

- The water-cement ratio is crucial. Too much water weakens the concrete because it creates more porosity as the excess water evaporates, reducing the density and strength. Too little water can hinder proper hydration, also reducing strength.

- What engineering principles support these differences?

- Hydration: Cement combines with water to generate C-S-H, the fundamental component that gives concrete strength. The amount of C-S-H generated is proportional to the amount of cement present (assuming sufficient water for the reaction).

- Water-Cement Ratio: This ratio determines the porosity of cured concrete. A lower water-cement ratio (within the feasible range) produces a denser, less porous construction with increased strength.

- What does this imply for mix design and curing practices?

- Mix Design: Optimizing cement content is crucial for achieving desired strength. The water-cement ratio must be carefully controlled to ensure adequate hydration without creating excessive porosity.

- Curing Practices: Proper curing (maintaining moisture and temperature) is essential to allow the hydration process to continue effectively, leading to higher strength development over time.

# PART F4 — Post-Hoc Analysis (if ANOVA is significant)

If your ANOVA result shows a significant difference, perform a post-hoc comparison using pairwise t-tests with Bonferroni correction.

### Steps:

- Run t-tests for each pair:

  - Group 1 vs Group 2

  - Group 1 vs Group 3

  - Group 2 vs Group 3

- Use Bonferroni-adjusted α = 0.05 / 3 = 0.0167

- Report significant differences and their engineering meaning.

### Engineering Interpretation Prompt:

- Which specific groups differ the most and why?

  - Based on our assumed p-values, the Low Cement group and the High Cement group show a statistically significant difference in concrete compressive strength. This suggests that increasing cement content from the "Low" range to the "High" range has a significant impact on the concrete's strength.

  - The difference is because a higher cement content provides more of the binding agent (C-S-H) that develops during hydration, leading to a stronger concrete matrix.

- Example for Age groups:

  - “Strength at 28 days is significantly higher than at 7 days because hydration progresses over time.”

# PART F5 — Application Scenario: Quality Control Report

Write a 1–2 paragraph engineering memo:

Scenario:
 Your concrete supplier wants to decide whether they should modify their mix design or curing process to achieve more consistent strength.

Using your ANOVA results:

- Tell the supplier whether strength varies significantly across groups.

- Explain why (based on real engineering principles).

- Recommend specific adjustments (cement content, water reduction, longer curing time, admixture adjustments).

# Student Deliverables (for ANOVA Section)

| Deliverable | Description |
| --- | --- |
| F1 | Defined groups + rationale |
| F2 | ANOVA table (Excel or Sheets output) |
| F3 | Statistical decision + engineering interpretations |
| F4 | Post-hoc comparisons (if needed) |
| F5 | Quality control memo |

# Rubric for ANOVA Section (50 pts)

| Criteria | Points |
| --- | --- |
| Group creation & justification | 10 |
| Correct ANOVA computation | 15 |
| Statistical decision-making | 5 |
| Engineering interpretation | 10 |
| Post-hoc analysis | 5 |
| Quality control memo | 5 |

# Deliverables

Submit:

- Scatterplots (8)

- Correlation matrix

- Interpretation of correlations

- Simple regression equation + prediction

- Multiple regression output + interpretation

- Recommendation memo (1–2 paragraphs)

# Rubric (100 pts)

| Component | Points |
| --- | --- |
| Scatterplots (clarity + analysis) | 20 |
| Correlation matrix + interpretation | 20 |
| Simple regression analysis | 15 |
| Multiple regression analysis | 25 |
| Engineering recommendation | 15 |
| Overall presentation | 5 |